TASK 1 :

Find Your Gateway IP:

- Windows: Open Command Prompt (press Win + R, type cmd, and hit Enter) and type:

Ipconfig

. Look for the Default Gateway under your active Wi-Fi or Ethernet adapter (commonly 192.168.1.1 or 192.168.0.1).

- Mac: Open Terminal and type netstat -nr | grep default or check System Settings > Network > Wi-Fi > Details.

Access the Page:

- Open your web browser and type [http://192.168.1.1](http://192.168.1.1) (or your specific gateway IP) into the address bar.

- You will see your router's login screen (e.g., TP-Link, Netgear, D-Link, or your ISP's custom firmware). Press PrtScn or use your device's screenshot tool to capture this page.

TASK 2 :

| Parameter | How to Find (Windows) | Example Output |
| --- | --- | --- |
| IPv4 Address | Run ipconfig in Command Prompt | 192.168.1.15 |
| Subnet Mask | Run ipconfig in Command Prompt | 255.255.255.0 |
| Default Gateway | Run ipconfig in Command Prompt | 192.168.1.1 |

TASK 3 :

Step-by-Step Instructions (Windows) :

- Press Win + R, type ncpa.cpl, and press Enter to open Network Connections.

- Right-click your Wi-Fi or Ethernet adapter and select Properties.

- Select Internet Protocol Version 4 (TCP/IPv4) and click Properties.

- Select Use the following IP address and enter:

  - IP address: 192.168.1.50 (or match your subnet, e.g., 192.168.0.50)

  - Subnet mask: 255.255.255.0

  - Default gateway: 192.168.1.1 (your router's IP)

  - Preferred DNS server: 8.8.8.8 or 1.1.1.1

- Click OK and test your connection by visiting any website (or running ping google.com in Command Prompt).

TASK 4 :

.  Log in to your router admin panel using your admin credentials.

. Navigate to Network, LAN, or DHCP Settings.

. You will see a setting labeled IP Address Pool or DHCP Range (e.g., 192.168.1.100 to 192.168.1.200).

.  Packet Dropping & Instability: Both devices will experience intermittent or total network failure because the router and switch cannot determine which device's MAC address should receive incoming data frames.

.  ARP Flapping: The router’s ARP (Address Resolution Protocol) table will constantly overwrite itself with conflicting MAC addresses, causing frequent disconnections for both devices until one is assigned a unique IP address.