Task 1:

Here is a comprehensive guide to help you execute, record, and document these hands-on network diagnostic exercises.

Task 1: Testing Local Device-to-Device Connectivity (Ping)

1. Step-by-Step Execution

- Ensure both your Laptop and your Friend's Phone are connected to the same Wi-Fi access point.

- Find the local IP address of your friend's phone:

  - Android: Go to Settings > Network & internet > Wi-Fi, tap the connected network name / gear icon, and look for IPv4 address (e.g., 192.168.1.45).

  - iOS: Go to Settings > Wi-Fi, tap the (i) icon next to your network, and check IP Address.

- Open Command Prompt on your Windows laptop (Win + R → type cmd → press Enter).

- Run the ping command pointing to the phone's IP address:

DOS

ping 192.168.1.45

2. Expected Output (Successful Ping)

Plaintext

Pinging 192.168.1.45 with 32 bytes of data:

Reply from 192.168.1.45: bytes=32 time=12ms TTL=64

Reply from 192.168.1.45: bytes=32 time=8ms TTL=64

Reply from 192.168.1.45: bytes=32 time=15ms TTL=64

Reply from 192.168.1.45: bytes=32 time=6ms TTL=64

Ping statistics for 192.168.1.45:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 6ms, Maximum = 15ms, Average = 10ms

Note on Client Isolation / ICMP Block: If you receive Request timed out despite both devices being on the same Wi-Fi network, your router may have AP Isolation / Client Isolation enabled, or the target device's software firewall is blocking ICMP requests.

TASK 2:

1. Command Execution

Open Command Prompt (Windows) or Terminal (Mac/Linux) and run:

- Windows:

DOS

tracert www.youtube.com

- Mac / Linux:

Bash

traceroute www.youtube.com

2. Sample Output Analysis

Plaintext

Tracing route to youtube.ui-t3.com [142.250.195.174]

over a maximum of 30 hops:

1    <1 ms    <1 ms    <1 ms  192.168.1.1 [Your Home Router]

2     2 ms     2 ms     2 ms  10.230.0.1 [ISP Gateway / OLT]

3     8 ms     7 ms     7 ms  172.16.20.1 [ISP Regional Core Router]

4    14 ms    13 ms    15 ms  72.14.212.1 [Google Edge Peering Node]

5    14 ms    14 ms    14 ms  142.250.195.174 [YouTube Server]

Trace complete.

- Result Analysis: In this sample trace, it takes 5 hops to travel from the local machine through the home router and ISP edge nodes to reach Google/YouTube's edge servers.

TASK 3 :

- Disconnect your friend's phone from Wi-Fi (turn off Wi-Fi or enable Airplane Mode).

- On your laptop, run the ping command again:

DOS

ping 192.168.1.45

Observed Error Messages & Explanations

| Error Message | Typical Cause | Explanation |
| --- | --- | --- |
| Destination host unreachable | ARP table failure / Device off-link | Your laptop's router or network adapter broadcast an ARP request asking "Who has 192.168.1.45?", but no device answered on the physical layer. |
| Request timed out | Packet dropped / Firewall block | The request was sent out over the network gateway, but no response (ICMP Echo Reply) was returned within the timeout period (4000 ms). |

TASK 4 :

To map out all local IP addresses on your network segment, open Command Prompt and execute:

DOS

arp -a

Sample Network Device Mapping Table

| IP Address | Physical (MAC) Address | Allocation Type | Device & Owner Identification |
| --- | --- | --- | --- |
| 192.168.1.1 | 00-11-32-4A-5B-6C | Dynamic | Primary Wi-Fi Router (Gateway) |
| 192.168.1.12 | 74-E5-F9-81-10-AB | Dynamic | My Laptop (Windows 11 PC) |
| 192.168.1.45 | A4-C3-F0-12-34-56 | Dynamic | Friend's Smartphone (Samsung Galaxy) |
| 192.168.1.100 | B8-27-EB-AA-BB-CC | Dynamic | Smart TV (Living Room) |
| 192.168.1.255 | FF-FF-FF-FF-FF-FF | Static | Subnet Broadcast Address |

TASK 5 :

====================================================================================================

PRACTICAL NETWORK DIAGNOSTICS & CONNECTIVITY REPORT

====================================================================================================

1. EXECUTIVE SUMMARY & SETUP OVERVIEW :

This report documents the configuration and diagnostic testing of a local wireless area network (WLAN).

The primary objective was to verify layer-2 and layer-3 connectivity between local client devices,

trace routing paths to public internet servers, and analyze network behavior when devices disconnect.

Network Infrastructure:

- Router / Gateway: Dual-Band Gigabit Wireless Router (Default Gateway: 192.168.1.1)

- Subnet Mask: 255.255.255.0 (/24 network)

- Test Devices: Windows Laptop (192.168.1.12), Smartphone (192.168.1.45)

2. LOCAL CONNECTIVITY TESTING (PING METHODOLOGY)

Both the Windows laptop and smartphone were associated with the same 2.4 GHz Wi-Fi SSID. Initial ICMP

Echo Requests were initiated from the laptop to the smartphone's local IPv4 address (192.168.1.45).

Results:

- Transmitted Packets: 4 | Received Packets: 4 | Packet Loss: 0%

- Average Round-Trip Time (RTT): 10ms

- Conclusion: Direct peer-to-peer ICMP communication succeeded across the wireless access point,

confirming AP Isolation was disabled on the router.

3. ROUTE PATH ANALYSIS TO PUBLIC WEB DESTINATION (TRACERT)

A layer-3 path discovery command (`tracert www.youtube.com`) was executed from the laptop terminal to

measure the number of routing hops and intermediate gateway latencies.

Results:

- Total Hops to Destination: 5 Hops

- Hop 1 (192.168.1.1): <1 ms (Internal LAN Gateway)

- Hop 2 (10.230.0.1): 2 ms (ISP Local Optical Line Terminal / POP)

- Hop 3–4: Intermediate ISP Edge Routers (7–15 ms)

- Hop 5 (142.250.195.174): 14 ms (Google Content Delivery Network Node)

- Conclusion: Packet pathing maintained minimal latency, taking 5 transitions to reach the target CDN.

4. DISCONNECTION DISRUPTIVE TESTING & ARP BEHAVIOR

To test failure states, the smartphone's Wi-Fi adapter was disabled, and ICMP requests were re-issued.

Observations:

- The terminal returned "Destination host unreachable" followed by "Request timed out".

- Technical Analysis: When a device leaves the network abruptly, the local host/router sends ARP

broadcast queries ("Who has 192.168.1.45?"). Since no target MAC address responds to register on

the Data Link layer, frame encapsulation fails and packets are dropped at the network layer.

5. NETWORK MAP (ARP DISCOVERY)

Executing `arp -a` revealed the local IP-to-MAC resolution table. All connected endpoints were mapped

successfully, confirming active DHCP dynamic leases managed by the central router.